Carry Park Security Process Flow

Firm Ware

Date: 27/May/2020

Prepared by: Cibin Baby, Indocosmo Systems

DATA FLOW

We have 2 Sequences

- Check-In Sequence

- Check-Out Sequence

Check-In Sequence data flow:

Carry Park BLE App sends connection request to CPOO1 Firmware and get connected

Press “CLOSE” Button in Carry Park BLE App

Carry Park BLE App sends “INIT (CLOSE)” command to CPOO1 Firmware

CPOO1 Firmware return a status response(S:N=0,S=1,L=0,P=0,B=69c0) to Carry Park BLE APP

Carry Park BLE App checks battery level of CPOO1 Firmware

CPOO1 Firmware create MSG2 using XYZ from the Updated string array

CPOO1 Firmware create MSG2 using XYZ from the Updated string array

Carry Park BLE App sends “KeyString”+MSG1+XYZ to Server.

CPOO1 Firmware picks X,Y,Z parameters and send to Carry Park BLE App

Carry Park BLE App sends the created string, i.e. MSG1 to CPOO1 Firmware.

Carry Park BLE App create string by selecting 4 random numbers characters from(0~9,A~Z,a~z).

CPOO1 Firmware checks the string received from Carry Park BLE App with the Hash Value already saved in the firmware

Carry Park BLE App sends the string received from Server to CPOO1 Firmware .

Server creates Hash Value.

CPOO1 Firmware creates Hash Value and save in CPOO1 Firmware.

Server composes “Key string”+ “MSG1” + “MSG2” and apply MD5 Hash algorithm.

CPOO1 Firmware composes “Key string”+ “MSG1” + “MSG2” and apply MD5 Hash algorithm.

Server sends Hash Value to Carry Park BLE App as a string-CLOSE(HASH) , i.e “C: <HASH VALUE >;”

If it is the same, CPOO1 Firmware sends Close(OK) , i.e “C:R=OK “ along with the Status Response ,  i.e “S:N=0,S=1,L=0,P=0,B=69c0” to Carry Park BLE App.

If it is Locked, Carry Park BLE App sends START command, i.e.”MM;” to CPOO1 Firmware. Command is to save Hash Value

CPOO1 Firmware sends response to Carry Park BLE App to show the lock is closed manually. i.e. “S:N=1,S=1,L=1,P=0,B=69c0”

Carry Park BLE App checks the lock is manually locked.

Carry Park BLE App sends STATUS Command, i.e. “st;” to CPOO1 Firmware.

User put baggage into Lock and Close the lock manually.

If it is OK, Carry Park BLE App displays as “Put baggage into Lock”.

Carry Park BLE App checks the status from CPOO1 Firmware and Confirm metal plate is OK.

CPOO1 Firmware sends STATUS response to Carry Park BLE App

Then Carry Park BLE App saves HASH Value for Open Process

Carry Park BLE App displays “Lock is done and Charging has started”

Carry Park BLE App displays “Lock is done and Charging has started”

Carry Park BLE App sends “Lock has become busy” to Server

Carry Park BLE App disconnects the connection automatically

Check-Out Sequence data flow:

Carry Park BLE App sends connection request to CPOO1 Firmware and get connected

Press “OPEN” Button in Carry Park BLE App

Carry Park BLE App sends “INIT (OPEN,HASH)” ,i.e “I:<HASH VALUE;>”command to CPOO1 Firmware

CPOO1 Firmware compares Hash Value with saved Hash Value and Send Status Response to Carry Park BLE App

Payment Process

Carry Park BLE App checks whether the lock is correct one.

Carry Park BLE App checks whether payment process occurred correctly.

CPOO1 Firmware create MSG2 using XYZ from the Updated string array

CPOO1 Firmware create MSG2 using XYZ from the Updated string array

Carry Park BLE App sends “CPOO1_Number”+MSG1+XYZ to Server.

CPOO1 Firmware picks X,Y,Z parameters and send to Carry Park BLE App

Carry Park BLE App sends the created string, i.e. MSG1 to CPOO1 Firmware.

Carry Park BLE App create string by selecting 4 random numbers characters from(0~9,A~Z,a~z).

CPOO1 Firmware checks the string received from Carry Park BLE App with the Hash Value already saved in the firmware

Carry Park BLE App sends the string received from Server to CPOO1 Firmware.

Server creates Hash Value.

CPOO1 Firmware creates Hash Value and save in CPOO1 Firmware.

Server composes “CPOO1_Number”+ “MSG1” + “MSG2” and apply MD5 Hash algorithm.

CPOO1 Firmware composes “CPOO1_Number”+ “MSG1” + “MSG2” and apply MD5 Hash algorithm.

Server sends Hash Value to Carry Park BLE App as a string-OPEN(HASH) , i.e “O: <HASH VALUE >;”

If it is the same, CPOO1 Firmware sends Open(OK) , i.e “O:R=OK “ along with the Status Response ,  i.e “ S:N=1,S=0,L=0,P=0,B=6940” to Carry Park BLE App.

Carry Park BLE App displays “Take the baggage out of Lock”.

Carry Park BLE App sends “Lock has become idle” to server and Lock opens automatically.

Then Carry Park BLE App clear the Hash Value.

CPOO1 Firmware sends Status response to Carry Park BLE App .  i.e.”S:N=1,S=0,L=0,P=0,B=6980”

Carry Park BLE App sends STOP command, i.e.”RR;” to CPOO1 Firmware. Command is to clear the Hash Value.

Carry Park BLE App checks the status from CPOO1 Firmware and Confirm metal plate is OK.

Carry Park BLE App disconnects the connection automatically

User takes out the baggage