Confidential Document

Disclosed Under 3 Party Agreement 17.06.2020

RAW DATA

The data sent by the Ethernet gateway to the server comprises of 13 bytes, or 26 nibbles.

- The first 6 bytes (12 nibbles) is the gateway Mac address.

- The next 4 bytes (8 nibbles) is the data from a single coin device.

- The last 3 bytes (6 nibbles) is the time the data was received.

4 Bytes (8 nibbles) Coin Data

The Coin data consists of four parts of 1 byte each i.e., Coin device ID, sensor ID, generated value of the sensor and last byte being zero for accelerometer, gyroscope, humidity sensors while temperature sensor sends float values upto decimal point 2.

The first byte is the device ID assigned to the Coin device (device ID can be anything from 1 to 255).

The 2nd byte is the sensor ID/Sensor threshold type. The different sensor threshold types currently supported are: 01,02,03,04,05,06,07,08,09,10,11,12,13,14,15 and 16.

| Sensor ID | Sensor Type |
| --- | --- |
| 01, 02 | Accelerometer Low and High |
| 03, 04 | Gyroscope Low and High |
| 05, 06 | Temperature Low and High |
| 07, 08 | Humidity Low and High |
| 09 | Temperature Stream – user defined periodic event (default every 20min if enabled) |
| 10 | Humidity Stream - user defined periodic event (default every 20min if enabled) |
| 11 | Status Message sent to server to notify whether a coin is active and running - The active status of the Coin sensors is sent once in every 24 hours if and only if there is no other data sent from the coin. |
| 13 | Battery Status – it is sent for the first time after 20 min when the coin is online, later it will only be sent when the battery of the coin has changed its status from high to medium or medium to low. |
| 12, 14, 15 | Accelerometer Stream for x, y and z axis respectively. [This actually is a separate project and supports accelerometer streaming, temperature and humidity sensors] |
| 16 | Dynamic ID - This will be sent at first when the Coin is online and whenever the dynamic ID of the Coin changes. |
| 27 | Acknowledgement for Set threshold, Set Stream & enable/disable sensors. |

The third byte is the sensor generated data for accelerometer, gyroscope and temperature, while the status message has a fixed byte DE sent; similarly battery status has 01 for high, 02 for medium and 03 for low.  While the dynamic ID sends the respective Dynamic Coin IDs depending on the positions of the Coins this can vary from 01 to 255(FF in hex).

3 Bytes (6 nibbles) Time Data

The time (UTC) data consists of 3 parts of 1 byte each i.e., hours, minutes and seconds in 24 Hour Format.

For Example, consider the Raw Data:  546C0E83350B01010C00080623

Breaking the data: 546C0E83350B   01010C00   080623

WHERE

546C0E83350B = Gateway Mac Address

01010C00 = Coin Data (01 – Device ID, 01 – Sensor ID (refer table), 0C – The actual value, 00 – considered only for temperature sensor and accelerometer stream data)

080623 = Time Data (08 – Hours, 06 Minutes & 23 Seconds) – Here 08, 06 and 23 are in Hex.

The above data has device ID 01, which means the data is coming from coin 01 of Gateway 546C0E83350B.

It has sensor ID 01 i.e., it is Accelerometer Low value (refer above table).

Actual Value – 0C (12 in decimal) => 12/10 = 1.2g

The data value of 1.2g for Accelerometer low from coin 01 of Gateway 546C0E83350B is received at 08:06:35 today.

Example 2: Temperature with decimal part.

Raw Data: 546C0E83350B01051B07080623 -> 546C0E83350B 01 05 1B 07 080623

WHERE

546C0E83350B = Gateway Mac Address

01051B07 = Coin Data (01 – Device ID, 05 – Sensor ID (refer table), 1B – The actual value, 07 – considered only for temperature sensor and accelerometer stream data)

080623 = Time Data (08 – Hours, 06 Minutes & 23 Seconds) – Here 08, 06 and 23 are in Hex.

The above data has device ID 01, which means the data is coming from coin 01 of Gateway 546C0E83350B.

It has sensor ID 05 i.e., it is Temperature Low value (refer above table).

Actual Value – 1B (27 in decimal) 07(7 in decimal) = 27.7

The data value of 27.7 degree Celsius for Temperature low from coin 01 of Gateway 546C0E83350B is received at 08:06:35 today.

Converting the actual value in readable format

| Sensor (Sensor ID) | Value(in Hex) | Conversion (to decimal) | Final Value |
| --- | --- | --- | --- |
| Accelerometer (g) (01 & 02) | 0C | 12 | 12/10 = 1.2 |
| Gyroscope (DPS) (03 & 04) | 0F | 15 | 15 * 10 = 150 |
| Temperature (degree Celsius) (05, 06 and 09) | 1B | 27 | 27 |
|  | 7E | 126 | 0 => If decimal value is 126, then temperature is 0 |
|  | 81 | 129 | -(129-126) = -3 => If decimal value is > 126, then temperature is -ve. |
| Humidity (%RH) (07, 08 & 10) | 3A | 58 | 58 |
| Accelerometer Stream (12, 14 and 15) | F825 | 63525 | 63525 – 65536 = 2011 Call function dataconvert(2011) Multiply the result with 0.000488. Final Value : 0.981368 |

Function dataconvert() for Accelerometer Stream calculation:   [This is written in PHP]

function dataconvert($dval)

{

$binsize = 15;

$size = 4;

$axis_data = $dval;

$sum = 0;

$b = array();

$bval = array();

$d = array();

$val = array();

if(($dval & 0x8000) == 0x8000)

{

$i=0;

$axis_data = (~$axis_data) + 1;

while($binsize != 0)

{

array_push($b, $axis_data & 0x1);

$axis_data = $axis_data >> 1;

$binsize--;

array_push($bval, $b[$i] * pow(2,$i));

$sum = $sum + $bval[$i];

$i++;

}

return(-1 * $sum);

}

else

{

$i=0;

while($size != 0)

{

array_push($d, $axis_data & 0xF);

$axis_data = $axis_data >> 4;

$size--;

}

array_push($val, $d[0]);

array_push($val, $d[1] * 16);

array_push($val, $d[2] * 256);

array_push($val, $d[3] * 4096);

for($i=0;$i<4;$i++)

{

$sum = $sum + $val[$i];

}

return($sum);

}

}

The data sent by the Server to Ethernet gateway comprises of 11 bytes, or 22 nibbles.

- Set Threshold

This is applicable for sensor IDs 01, 02, 03, 04, 05, 06, 07 and 08.

To set the threshold for a sensor, use the following format:

<gateway_id><device_id><sensor_id>00<threshold><dynamic_id>

| <Sensor> | Allowed Values | <threshold> | Example |
| --- | --- | --- | --- |
| 01 & 02 | 0.001 to 15.875 (0.001, 0.1, 0.375, 0.5, 0.625 and so on with further thresholds in multiples of 0.125 upto 15.875) | - If threshold = 0.001, then <threshold> is 01. - If threshold = 0.1, then <threshold> is 02. - If threshold > 0.001 and threshold < 0.1, then <threshold> is 01. - If threshold > 0.1 and threshold < 0.375, then <threshold> is 02. - If threshold >= 0.375, then <threshold> is <threshold> multiply by 8 and then convert to Hex. | If accelerometer low is to be set to 0.1, then the <threshold > will be 02(in Hex) If accelerometer low is to be set to 4.25, then the <threshold > will be 4.25 * 8 = 34 => 22 (in Hex) So final data will be 546C0E83350B0101002201 |
| 03 & 04 | 10 to 1990 (multiples of 10) | Divide by 10 and then convert to Hex. | If Gyroscope low is to be set to 150, then the <threshold> will be 150/10 = 15 => 0F (in Hex). So final data will be 546C0E83350B0103000F01 |
| 05 & 06 | -39 to 124 (integer) | Convert to Hex. If <threshold> = 0, then <threshold> is 126 and then Convert to Hex. If <threshold> is -ve, then add 126 to the <threshold> and then Convert to Hex. | If threshold is to be set to 25, then <threshold> will be 25 => 19(in Hex.) If threshold is to be set to 0, then <threshold> will be 0 + 126 = 126 =>7E (in Hex.) If threshold is to be set to -5, then <threshold> will be 5+126=131 => 83(in Hex) |
| 07 & 08 | 2 to 99 (whole number) | Convert to Hex. | If Humidity low threshold is to be set to 20, then <threshold> will be 20 => 14 (in Hex) So final data will be 546C0E83350B0107001401 |

- Get current Value

This is applicable for sensor IDs 01, 02, 03, 04, 05, 06, 07 and 08.

To get the current value of the sensor for a particular coin of a gateway, use the following format:

<gateway_id><device_id><sensor_id>0000<dynamic_id>

- Set Stream

This is applicable for sensor IDs 09, 10 and 12.

To set the stream rate for temperature, Humidity and Accelerometer Stream, use the following format:

<gateway_id><device_id><sensor_id><format><rate_val><dynamic_id>

WHERE

| Format Type | <format> | <rate_val> | Example |
| --- | --- | --- | --- |
| Seconds | 15 | Allowed 20 to 59. Convert to Hex to get <rate_val>. | To set 20 Seconds temperature stream for Coin 01 of Gateway 546C0E83350B, <rate_val> is 14 (20 converted to hex). So final data will be 546C0E83350B0109151401 |
| Minutes | 16 | Allowed 1 to 59. Convert to Hex to get <rate_val>. | To set 1 Minute temperature stream for Coin 01 of Gateway 546C0E83350B, <rate_val> is 01 (1 converted to hex). So final data will be 546C0E83350B0109160101 |
| Hours | 21 | Allowed 1 to 96. Convert to Hex to get <rate_val>. | To set 2 Hours temperature stream for Coin 01 of Gateway 546C0E83350B, <rate_val> is 02 (2 converted to hex). So final data will be 546C0E83350B0109210201 |

- Enable/Disable sensor

By default all the sensors are enabled on the coin. To disable or re-enable use the following format:

<gateway_id><device_id><sensor_code><activationCode>00<dynamic_id>

WHERE

| Activation Type | <activation code> |
| --- | --- |
| Disable | 44 |
| Enable | 45 |

| Sensor Type | <SensorCode> |
| --- | --- |
| Accelerometer | 41 |
| Gyroscope | 47 |
| Temperature | 54 |
| Humidity | 48 |
| Temperature Stream | 74 |
| Humidity Stream | 68 |
| Accelerometer Stream | 61 |

ACKNOWLEDGEMENT

Acknowledgement for Set Threshold, Set Stream and Enabling/Disabling sensors are received from the devices to the server with ID 27.

Format: <gateway_id><device_id>27<sensor_id><value><time part>

Example 1: 546C0E83350B 01 27 54 44 080623  Acknowledgement received for Disabling Temperature sensor for Coin 01 of Gateway 546C0E83350B.

Example 2: 546C0E83350B 01 27 09 14 080623 Acknowledgement received for Setting Stream for Temperature for Coin 01 of Gateway 546C0E83350B.

IndyGo Co. Ltd.